ACDC JetRacer Pro

Field Procedure & Command Reference

Bartek Broclawik  |  NJIT ACDC  |  March 2026

# 1. System Overview

The JetRacer Pro runs a full perception-to-actuation pipeline on a Jetson Nano 4GB. The camera feeds frames into a traffic light detector, which outputs RED / GREEN / UNKNOWN. That signal drives the ESC via a PCA9685 PWM controller over I2C bus 1.

File structure on the Jetson:

/home/jetson/run.py                                    ← integration entry point

/home/jetson/jetson/ssunc_perception/

traffic_light_detector.py                         ← perception pipeline

# 2. SSH Into the Jetson

## 2.1 Finding the Jetson IP Address

On the Jetson keyboard (or terminal), run:

hostname -I

This prints the current IP address. Example output: 172.20.10.13

NOTE: On campus WiFi, device-to-device SSH is blocked. Always use a personal phone hotspot. Connect both your Mac and the Jetson to the hotspot, then get the IP.

## 2.2 Connecting the Jetson to a Hotspot

On the Jetson terminal, connect to your hotspot:

nmcli dev wifi connect "YourHotspotName" password "YourPassword"

Then confirm the IP with:

ifconfig wlan0

Look for the inet address — that is the IP to use for SSH and SCP.

## 2.3 SSH Command (from Mac)

ssh jetson@172.20.10.13

Password: jetson

NOTE: Replace 172.20.10.13 with the actual IP from hostname -I. The IP changes every time the Jetson reconnects to a network.

# 3. Copying Files to the Jetson (SCP)

SCP (Secure Copy) transfers files from your Mac to the Jetson over SSH. Run from a Mac terminal window — not from inside the SSH session.

Copy a single file to the Jetson home directory:

scp run.py jetson@172.20.10.13:~/

Copy a file to a specific folder:

scp traffic_light_detector.py jetson@172.20.10.13:~/jetson/ssunc_perception/

Copy an entire folder from Jetson to Mac:

scp -r jetson@172.20.10.13:~/jetson/ssunc_perception ~/Desktop/

NOTE: If the file path has spaces (e.g. RC Car), wrap it in quotes: scp "RC Car/run.py" jetson@...

# 4. Running the Integration

## 4.1 First Run Checklist

- Jetson is powered on and connected to hotspot

- Camera CSI cable is seated firmly

- ESC and servo are powered (battery connected)

- Car is on the ground pointed at the traffic light

- SSH session open from Mac

## 4.2 Set Max Power Mode

Switch Jetson to 10W MAXN mode for full CPU/GPU performance:

sudo nvpmodel -m 0

Verify:

sudo nvpmodel -q

## 4.3 Set Display (required over SSH)

Before running any script that opens a camera window over SSH:

export DISPLAY=:0

## 4.4 Run the Integration

python3 ~/run.py

Expected startup sequence:

[init] MODE1=0x21  PRESCALE=121

[init] PCA9685 verified OK.

[main] ESC armed.

[main] Camera opened.

[main] Detector ready. Entering control loop.

After startup, the car will hold neutral on RED or UNKNOWN, and move forward on GREEN.

Stop safely with Ctrl+C — ESC returns to neutral automatically.

# 5. Camera Troubleshooting

## 5.1 Restart the Camera Daemon

If the camera fails to open or gives nvargus errors, restart the daemon:

sudo systemctl restart nvargus-daemon

Then rerun the script.

## 5.2 Test the Camera Pipeline Directly

gst-launch-1.0 nvarguscamerasrc ! nvvidconv ! videoconvert ! xvimagesink

If a window opens with camera feed, GStreamer and the camera are working correctly.

## 5.3 Kill Lingering Camera Processes

sudo pkill -f nvargus

Run this if the camera is stuck from a previous session.

# 6. Quick Command Reference

| Command | Description |
| --- | --- |
| ssh jetson@<IP> | SSH into the Jetson from Mac |
| hostname -I | Get Jetson IP address (run on Jetson) |
| ifconfig wlan0 | Check WiFi IP and connection status |
| scp file.py jetson@<IP>:~/ | Copy file from Mac to Jetson home dir |
| scp -r jetson@<IP>:~/folder ~/Desktop/ | Copy folder from Jetson to Mac |
| python3 ~/run.py | Run the integration script |
| export DISPLAY=:0 | Enable GUI windows over SSH |
| sudo nvpmodel -m 0 | Set Jetson to 10W max power mode |
| sudo nvpmodel -q | Check current power mode |
| sudo systemctl restart nvargus-daemon | Restart camera daemon |
| sudo pkill -f nvargus | Kill stuck camera processes |
| gst-launch-1.0 nvarguscamerasrc ... | Test GStreamer camera pipeline |
| rm ~/run.py | Delete old run.py before uploading new one |
| grep -n 'class' file.py | Find class names in a Python file |
| cat file.py | Print contents of a file to terminal |

# 7. Hardware Constants (Do Not Change)

| Command | Description |
| --- | --- |
| I2C Bus | Bus 1 only — bus 0 is dead on this hardware |
| PCA9685 Address | 0x40 (confirmed via i2cdetect -y -r 1) |
| CH0 | Steering servo |
| CH1 | ESC / throttle |
| Prescale | 121 → 50Hz PWM |
| STEER_CENTER | 320 |
| STEER_RIGHT | ~280 (soft limit) |
| STEER_LEFT | ~370 (soft limit) |
| ESC_NEUTRAL | 307 — always return here on exit |
| ESC_FORWARD_MIN | 320 — first ground movement |
| ESC_FORWARD (demo) | 325 — safe creep speed |